Garment Manufacturing Process: Step by Step Section Wise

17/08/2020 by textileblog

Garment manufacturing is a huge process. All operations are done in different departments in garment industry. It is not possible to describe briefly about apparel production. Garment manufacturing is a sequential processes such as designing, sampling, laying, marking, cutting, stitching, checking, finishing, pressing and packaging etc. In this process, raw materials convert into finished products. I will describe all operations of garment manufacturing step by step in section wise.

Flow Chart of Garment Manufacturing Process:

Design

↓

Sample

↓

Pattern

↓

Pattern Grading

↓

Marker Making

↓

Spreading Fabric

↓

Cutting Fabric

↓

Sorting out & bundling cut fabric

↓

Print/ Embroidery

↓

Assembling the part by sewing

↓

Sewing quality inspection

↓

Ironing/ Pressing

↓

Hand Tag/Label

↓

Final Inspection

↓

Packing

↓

Size wise cartooning

↓

Cartoon Inspection

↓

Warehouse

↓

Shipment

Garment manufacturing process in different departments of garment industry are described below:

Sample Section:

There are many types sample are needed when garments are going to make a product.

Types of sample:

First pattern sample

Development sample

Second pattern sample

Counter sample

Salesman sample

Photo sample

Approval sample

Pre-production sample

Production sample

Shipment sample

Types of sampling with their details:

1. First pattern sample:

A design given by buyer and make it physical version of the garment.

Here are some criteria to develop a first sample. They maintain sequence to develop it.

Thinking of a design

↓

Draw in CAD

↓

Pattern

↓

Sample

2. Development Sample:

Which sample are developed by sample section from primary pattern sample.

3. Second Pattern Sample:

Usually designer/ developer always ask for some changes to the first pattern. Second pattern is made as per comments.

4. Counter Sample:

Where first pattern is made on designers artwork, Counter sample is to make not on designer’s artwork, has to follow another sample given by the merchandiser.

5. Salesman Sample:

Salesman Sample is made when PRICE is confirmed and order are on speculation. Buyer held a meeting with its customer and record their response on order quantity per COLOR, SIZE etc. and finally plac order to their vendor.

6. Photo Sample:

Photo samples are made with actual color and material to be worn by models on the event of SHOOTING for catalog.

7. Approval Sample:

It is sent to buyer for his APPROVAL of the conformity that the revision is done correctly. If any fault found then the sample go back to garments.

8. Pre-Production Sample:

When the production accessories come to the garments for the production then garments makes a sample for a buyer. Which is called pre-production sample.

9. Production Sample:

Assurance to the buyer that the bulk is being produced as per specifications.

10. Shipping Sample:

This sample send to shipping inspector. Who give the product to the buyer. It is important sample because of shipping inspector give a note to buyer as sample base.

garment sample section

Fig: Garment sample section

Sample making procedure:

There are many types of procedure to make sample. But in my garments, they follow this type procedure. I am giving a flow chart to making sample in my garment manufacturing industry.

Thinking and Sketch

↓

Material and 1st Pattern

↓

Photo type sample and give the sample to buyer

↓

Take the buyer comments

↓

Dummy

↓

Approval For Production

↓

Take the buyer comments

↓

Bulk production

↓

Send Photo, Sales man, shipping sample

↓

Production Sample

↓

Send to the Garments

Cutting Section:

Fabric is a single piece of sheet. When we need garments then we need to cut. Cutting is final step of sample, marker and fabric spreading. Because next process is sewing. When sewing starts then other process also starts.

There are many types of cutting process but in Bangladesh almost everyone uses a same process. And this is manually process. Cutting master cut the fabric by hand cutting machine. There is a big long table where cut the fabric.

garment cutting section

Fig: Garment cutting section

Maker paper kept up on the fabric surface, and cutting muster follow the line and cut the fabric.

Pattern Making Section:

Pattern making process is drawing or art in a flat paper of flat fabric. Basically, garment manufacturing industries make their pattern by using flat paper. Because pattern can easily removable from pattern paper. So, garments industries follow the pattern making process by using paper. In Divine Textile Ltd, they also make their pattern by using paper. And the paper name is pattern paper.

Pattern is important because, it is primary stage of a garments making. If pattern is accurate then the garments should be accurate. Buyers give the size list to the garments, then the garments follow the size and make the pattern with allowance. Pattern is the big issue for every garment.

Marker Making Section:

Marker is a process where every pattern is kept in a long fabric. And this process factory minimized the wastage of the fabric before cutting. Marker increase the efficiency of the fabric.

Types of marker making:

Manually marker making process

Computerized marker making process

1. Manually marker making process:

This is a process where a man done the maker by manually. No computer software is used them. Basically, who do this process he will be very experienced person, and he know how to do this. In my garments the maker making process is manually.

2. Computerized marker making process:

This is hundred percent done by computer, by using CAD (Computer aided design) or many others software.

Effect of Marker Making:

Increase efficiency

Minimize fabric wastage

Maximized using of fabric

Fabric Spreading Section:

In this process fabric laid on a big table which is used for cutting. Fabric spreading is important because of if fabric do not lay properly then outcome product (after cutting) was not correct. So, when the fabric go for production then its make difficulties in making garments. So proper fabric spreading is important for garment manufacturing industry.

When fabric lay properly, they used one kind of clip (Fabric attaching clip) for fixed the fabric. Marker paper also keep up on the fabric surface. Then they attach the fabric with clip.

Types of fabric spreading system:

Automatic

Manual

1. Automatic:

When fabric spread by machine on the cutting table then its called automatic fabric spreading.

2. Manual:

When fabric spread by man on the cutting table then its called manual fabric spreading. In my factory they work in manually.

Cutting Process:

Marker Making

Fabric Spreading

Marker placement

Attaching with clip

Cut the fabric

Numbering

Bundling

Heat Cutting system:

When we need to cut non-woven fabric then normal cutting machine is not suitable for cutting, because non-woven fabric is basically slippery then other fabric. So we need to heat cutting machine.

Types of cutting process

Manual Cutting

Heat Cutting

Lesser Cutting

1. Manual Cutting:

It is a process which done by man is called manual cutting process. In my garments they cut the fabric in manual process. Cutting master give the instruction to worker and worker follow the instruction.

2. Heat cutting:

Basically, a heat machine is used for this process and the machine has copper coil. And electricity passing the coil and it became hot. Then it cut the fabric. This machine is not suitable for cloth type fabric, it is used for non-woven fabric.

3. Lesser Cutting:

A lesser light is used for cutting. It is very expensive cutting method but cutting is very effective.

Sewing Section:

Sewing is big important part of a garment’s product. Without good sewing we cannot introduce our products in worldwide. Sewing is a process where two pieces of fabric are join. Many factors are related with sewing. (i.e.: Needle size, Machine type, Thread count, Worker capacity, Production line etc.) So sewing section is not only fabric join, it is core part of a garment manufacturing industry.

garment sewing section

Fig: Garment sewing section

Elements of sewing:

Sewing Machine

Needle

Thread/Yarn

Sewing Machine:

There are many types of machines are used in Divine Textile Ltd. Every types of sewing machines are available here. They (my garments) produce jackets, trousers etc so they need heavy machines for sewing.

List of sewing machine:

Plain Machine

Overlock machine

Bar tack machine

Button hole machine

Button attaching machine

Feed of the arm

Kansai

Discuss About Sewing Machine:

1. Plain machine:

Plain machine is basically use for attach two fabric. For long stitch this machine is use. There are many brands in plain machine. Some is manual and some is automatic. But every machine works same thing.

Brands:

Juki

Hakkari

Brother

JUKI is a well-known brand in sewing machine. Every company used this machine. Initial power of machine is very good. This machine origin is China. This machine is manual machine. Operators need set the machine, because settings are going change when products are changed.

HIKARI is also well-known brand in sewing machine, and this machine is semi-automatic machine. If one system is input in this machine then no need gives same settings once again. Every data is stored in this machine. Production rate are automatically calculated in this machine.

BROTHER is a Chinese brand, which produce many textile and other machineries. They made manual, semi-automatic, fully automatic machines for sewing. But in my garments, they have used manual machine. This machine is manual but this machine is safe for human. Brother ensure the workers safety. They introduce many devices in this machine for workers safety.

You may also like: Sewing Machine Types, Features and Functions

Example: if any needle breakdown then machine automatically gives sound. There are many types of facility in this machine.

2. Overlock machine:

An overlock is a kind of stitch that sews over the edge of one or two pieces of cloth for edging, hemming, or seaming. It is very effective machine for garments good, because without overlocking edge finishing is not to be good. There are many brands available in market but Divine Textile Ltd. use only one brand for overlock.

3. Bartack machine:

Bartack machine specially use for lock stitch. It contains a needle and a bobbin. It produce high density lock stitch in short place. In my garments they use bartack machine to stitch the side of pocket in jacket and pants. Stitch density depends on the GSM of fabric. If the GSM is high then the stitch density is high, If GSM is low then stitch density is also low.

4. Overlock machine:

This machine basically used for protect the edge of fabric and trim the extra part of fabric. This machine used when product is done by plain stitch. Five thread are used in overlock stitch machine. A sharp blade also attaches with the machine. This sharp blade cut the extra part of the fabric and gives a plain finishing in the garment’s products. This machine is very fast machine so need good experienced worker to run this machine, otherwise any types of accident can be happened.

5. Button attaching machine:

This machine basically used for attach button on the jackets or garments goods. It is very high speed machine and this machine can attach up to 60+ button in a minute. In my garments there are two types machine. One is plastic or normal button attaching machine another is steel or clip or tip button attaching machine.

6. Feed of the arm:

Feed of the arm machine is actually a chain stitch machine for chain stitch designs working with sew and stitch. It contains looper instead of bobbin. By this machine, multi thread chainstitch can be produced on garments. By this machine, stitch can be produced on heavy fabrics, like denim or jeans.

You may also like: Study on Quality Control in Sewing Section

Discuss About Needle:

Needle is an essential product for sewing. Only machine cannot sew without needle. Needle made by stainless steel with sharp head. In sewing machine, there are different types of needle is used. In my garments industry I was found three types of needle. Which is used on plain, overlock and bartack machine. Some needles are quite bigger usually it depends on fabric. If fabric is courser then need courser needle.

Needle varies machine to machine:

In JUKI sewing machine the needle name is BP. And needle size is 9,11,14,16,18,20

In Sunstar sewing machine they used, DP, and needle size is 9,11,14,16,18,20

In Overlock machine, the needle size is small then stitch machine. Needle size of overlock machine is DC, 9,11,14,16,18,20

Sewing Process:

In my industry they follow a process to maintain sewing.

↓

Collect fabric from cutting section

↓

Input the fabric into the line

↓

Set the machine settings

↓

Post production meeting

↓

Production line up set

↓

Production Starts

↓

Check goods quality by Sewing Quality Inspector

↓

Fault return to the worker

↓

Output

Quality Section:

Every garment ensures the quality first, because if garments are unable to ensure the quality then that garments cannot find good consignment in future. In Divine Textile Ltd, 60 people are worked in finishing. 20 workers are finishing quality inspector, 20 workers are sewing quality inspector, 10 workers are assistant finishing quality inspector and 10 workers are assistant sewing quality inspector. And this garments also has inspection room. They provide many high-tech machines and Quality manager for each buyer.

Garment inspection

Fig: Garment inspection

Work of Sewing Quality Inspector:

Sewing Quality Inspector is a person who check the quality of garments when production is running in the industry. He/She check the product and detect the fault of the product. If any fault was found then the inspector directly sends to the senior sewing machine operator and order to remove the fault from the product. Basically he or she is work for online quality control.

Work of Finishing Quality Inspector:

Finishing Quality Inspector is a person who check the garments products when sewing inspector transfer to the finishing line. Basically, it is offline quality control and maximum fault was removed. They check only size label and critical defects (broken needle, insects etc.). They use metal detector in every product.

Finishing Section:

Finishing department is the department which comes after all the department & it plays an equal important role in the final appearance of the garments. This department includes majorly of the following steps.

garment finishing section

Fig: Garment finishing section

Thread Sucking

Remove the extra tread from the garments, and clean the product.

Ironing

Ironing is the use of a heated tool (an iron) to remove wrinkles from fabric. Give the good finishing in the product. They use steam iron in the jacket. Steam helps to smooth finishing. Temperature of iron is 80-90 degree Celsius.

Packing Section:

After the passing quality section, our garments are ready for shipment. Before shipment we need to pack our product. In hare 30 workers and 5 packing managers are work for packing. Packing style varies buyer to buyer. Then they follow the instructions and pack the goods for the shipment.

garment packing section

Fig: Garment packing section

It is easy work but not so easy. First, they pack the product in poly-pack and then they put all garments in the cartoon. Cartoon size is also given by buyers. And how many pieces are kept in a one cartoon also give in the order sheet.

Store:

A place where many accessories, trims, fabric and many important elements of garments are kept. Basically, store are two types, one is normal store and another machine store.

1. Normal Store:

A place where every types of office equipment’s, production tools, accessories and trims are kept. Every necessary product of garments is stored here. A data man entry the data of the product, and give the product when anyone need.

List of the product in the normal store:

Paper

Pen

Office Documents

Previous swatch card

Button (any types)

Fabric (any types)

Label

Token

Office stationery

Needle

Thread

Previous Buyers Data

Zipper

Garments Accessories

Garments trims

2. Machine Store:

A place where machine instruments are stored. Every tool of sewing machines are kept here. When any parts need to change then the mechanic come here and replace the parts from the machine.

List of the product in machine store:

Arm

Balance Wheel/Hand Wheel

Bed

Bobbin Case

Bobbin Cover

Bobbin Winder

Face Plate

Feed Dogs

Head

Needle bar

Needle Clamp

Pattern/Stitch selector

Presser Foot

Presser Foot Lever

Reverse Lever

Slide Plate

Spool Pin

Spool pin for bobbin winding

Stitch regulator

Take up Lever

Tension Disc

Thread Cutter

Thread Guide

Throat Plate or Needle Plate

Merchandising Section:

Merchandising is an important section in every garment manufacturing industry. In merchandising section, they communicate with buyers and deals with buyers, and they pricing the product also they works in shipment too.

Simply merchandiser works with the whole productions of garment manufacturing. He maintains every single section by own risk. It is very challenging department in garment manufacturing industries.

garment merchandising section

Fig: Garment merchandising section

Works of a merchandiser in garment manufacturing industry:

Contact with buyers

Invite the buyers in the factory

Buyers visit the garments

Buyers gives the audit sheet to the merchandiser

Resolve the problem, if any fault found

Take the product list from the buyers

Pricing the products

Send to the buyers

Fix the pricing from the both side (factory and buyers)

Take the sample sheet from the buyer (Primary order sheet)

Send to the sample sheet to the sample section

Send the sample to the buyer

Buyers approval

Give the main order sheet to the merchandiser

Create Latter o credit (master L/C)

Merchandiser create back to back L/C, where accessories comes from.

Send a production sample and photo sample of the garments

Buyers approval

Swatch card making

Swatch card approval

Take the full production

Production follow up

Maintain TNA

Audit (when production running)

Transportation booking

Cartoon booking

Follow up finishing

Follow up packing

Shipment the garments

Take the full money from the buyers.

Send the order sheet in the store (for future)

This is the basic work which I observe in the factory, but they do also some critical works too, like fabric inspection, machine inspection, compliance inspection etc. Those works are internal, as a I’m just an intern in this garment manufacturing industry, so I have some restriction to observe those works.

They work lot in technical and official. And they work for garments and also work for country.

You may also like: How to Be a Good Merchandiser in Garments

Hierarchy of Merchandising:

In Divine Textile Ltd, they follow this type hierarchy in merchandising.

Director of Merchandiser

↓

Senior Merchandiser

↓

Merchandiser Officer

↓

Junior Merchandiser

↓

Assistant Merchandiser

↓

Trainee Merchandiser

↓

Intern

Compliance Section:

Divine Textile Ltd is a compliance factory in Gazipur, Dhaka. They ensure the worker safety and rights of the workers. They maintain the code of conduct (coc). They observe the everything in the garments manufacturing and maintain the garments.

Compliance works for workers right, so they ensure the legal rights of the worker. There are many corporations works for compliance. BSCI, WRAP, BOND etc. Compliance audit report helps in order. Basically, buyers asked for send the compliance report to them. Compliance report varies country to country. If the buyers are Europe so they want BSCI report. If the buyer are America then they asked for WRAP report. Divine Textile Ltd. target country in Europe so they only focused BSCI report.

Rules of compliance:

Compensation for holiday

Leave with wages

Health register

Time care

Accident register

Workman register

Equal remuneration

National festival holiday

Overtime register

Labor welfare

Weekly holiday fund

Sexual harassment policy

Child labor abolition policy

Anti-discrimination policy

Zero abasement policy

Working hour policy

Hiring /recruitment policy

Environment policy

Security policy

Buyers code of conduct

Health and safety committee

Canteen

You may also like: Compliance Issues and Audit Requirements of Two Different Garments Factories

Health:

Drinking water at least 4.5 L/day/employee

Cup availability

Drinking water supply

Water cooler, heater available in canteen

Drinking water signs in native language and English locate minimum 20 feet away from

Work place:

Drinking water vassal clean at once in a week

Water reserve at least once a week

Water center in charge person with cleanliness

Suggestion box register

Toilet:

Separate toilet for women and men

A seat with proper privacy and lock facility

Urinal accommodation

Effective water sewage system

Soap toilet

Water tap

Dust bins

Toilet white washed one in every four months

Daily cleaning log sheet

No-smoking signs

Ladies /gents toilet signs both in native language and English

Deposal of wastes and effluent

Fire:

Sufficient fire extinguisher and active

Access area without hindrance

Fire signs in both languages

Fire certified personal photo

Emergency exit

Safety Guard:

Metal glows on good condition.

Rubber mats & ironers

First aid box one

Ironers wearing sleepers

First trained employees

Motor/needle guard

Eye guard

Nurse

Doctor

Medicine

Medicine issuing register

Welfare officer

Others Facility:

Room temperature

Lighting facilities

CategoriesApparel Industry

All Over Printing (AOP): A Fast Growing Sector in Bangladesh

Yarn Count System and Determination by Different Methods

3 thoughts on “Garment Manufacturing Process: Step by Step Section Wise”

Rory Ganz

20/10/2020 at 12:39 pm

These are used by a variety of different people in different settings, so it’s important that you know what to look for when searching for metal detectors. Once you know what you are looking for, it should be easier to narrow down your search for the best metal detector for your needs. You should also keep your prices in mind, so that you don’t overspend.

Reply

Onnto

26/05/2021 at 8:13 pm

Hmmm .. this is an interesting process of garment production. All the processes have been highlighted step by step and the most interesting step was the heat cutting process.

Reply

Wintex Fabric and Fashions

17/05/2023 at 12:48 pm

Really! it’s legal information of Readymade Garments manufacturing.

Reply

Leave a Comment

Comment

Name

Name *

Email

Email *

Website

Website

Save my name, email, and website in this browser for the next time I comment.

Notify me of follow-up comments by email.

Notify me of new posts by email.

This site uses Akismet to reduce spam. Learn how your comment data is processed.

Ezoicreport this ad

Search for:

Search …

Subscribe for Latest Posts

Email Address

Email Address

Subscribe

Textile Learner

Sponsored

ChiuVention

Sponsored

Ottoman Textiles: Wholesale & Hotel Bedding Suppliers

Trending Posts!

100 Different Types of Fabric and Their Uses100 Different Types of Fabric and Their Uses

Different Types of Lingerie with PicturesDifferent Types of Lingerie with Pictures

Pantone Color Guide in Textile Industry | TCX & TPX | Pantone NumberPantone Color Guide in Textile Industry | TCX & TPX | Pantone Number

Textile Manufacturing Processes for Students and ProfessionalsTextile Manufacturing Processes for Students and Professionals

Garment Manufacturing Process: Step by Step Section WiseGarment Manufacturing Process: Step by Step Section Wise

List of Textile Colleges and Universities in BangladeshList of Textile Colleges and Universities in Bangladesh

4 Point System for Fabric Inspection with Example4 Point System for Fabric Inspection with Example

Importance of Marketing in Garment IndustryImportance of Marketing in Garment Industry

Men's Pants Pattern Drafting from MeasurementMen's Pants Pattern Drafting from Measurement

Warping Process in Weaving: Objectives, Types, Operation and ProblemsWarping Process in Weaving: Objectives, Types, Operation and Problems

Ezoicreport this ad

Ezoicreport this ad

Recent Posts

Performance Measurement Parameters in Cutting Department

Strategies for Effective Essay Writing and Time Management for Students

Making a Quality Report of a Batch of Finished Knit Fabric

Operation Breakdown and SMV of a Trouser

Stencil Printing: Definition, Process and Uses

Revolutionizing Manufacturing with Data Analytics and Business Intelligence

Care Label Symbols for Clothing and Textile Products

Production Organization Systems in Garment Industry

How to Calculate Production Capacity of a Garment Factory

Line Balancing in Garment Industry

Categories

Accessories

Apparel Industry

Automation

Campus

Career

Clothing

Defects

Denim

Dyeing

ERP Software

Fabric

Fashion Industry

Fiber

Garment Wash

Industrial Engineering

Knitting

Merchandising

Nanotechnology

Nonwoven

Pretreatment

Printing

Quality Control

Sewing Machine

Spinning

Sustainable

Technical Textile

Testing

Textile Finishing

Textile Industry

Thesis

Underwear

Weaving

Wet Processing

Yarn

Install Textile Blog Apps